Business Requirements Document

Chilling Centre Milk Collection — Native Windows Application

Version 6.0  •  September 2026  •  Quality Decision Clarified — Manual, Not Automatic

Changelog (v2.1): Confirmed this application targets the Milk Chilling Centre (CC/MCC) tier, not Bulk Milk Cooler (BMC)/farmer-procurement software — see §20. Added a reviewed landscape of comparable existing software (§21). Marked chilling-tank tracking, bulk storage tank telemetry, CIP and plant/equipment monitoring explicitly out of scope, by decision (§22). Confirmed outbound tanker dispatch and reconciliation as in-scope but sequenced Post-MVP (§23), not part of the MVP list in §17.

Changelog (v2.2): Outbound tanker dispatch and reconciliation/closing-stock reporting moved from Post-MVP (§23) into MVP (§17) — estimated at roughly 2-3 developer-weeks combined, reusing the idempotent-create and durable-outbox patterns already built twice (reception, then override). Dispatch's composite FAT/SNF is computed as a weighted average of receptions since the last dispatch — no tank sensor involved, consistent with §22 — working assumption, pending confirmation. §22's out-of-scope list and the deferred lab-test note under §10 are unchanged.

Changelog (v3.0): BRD validated against real Karnataka/Tamil Nadu evidence (TUMUL's full ERP RFP, BAMUL's operating scale, Aavin's live-deployed stack) across both a private-CC lens and a cooperative-federation lens - see §24. Five additions approved and added to MVP: CIP compliance logging (record-keeping only, not tank/equipment telemetry - §22 unchanged on that point), device calibration tracking with due-date alerts, an optional parent-BMC/route field on Source, a read-only rate-chart reference at reception (no farmer-payment computation), and a notification-hook abstraction (SMS gateway wiring itself deferred). Dispatch gains tare/gross tanker weight fields; Reconciliation gains FAT/SNF variance tracking, not just quantity. Offline-media (USB) sync fallback and building a real SMS send-path were both explicitly considered and excluded from MVP - see §24.

Changelog (v4.0): MVP narrowed by explicit business decision to eight components: Milk Arrival, Reception & Testing, Quality Decision, Local Save, Held in Chilling Centre (physical stage, not software-tracked), Rate Chart Reference, Notification Hook, and Cloud (RBAC/Audit/Dashboard). Outbound Tanker Dispatch, Reconciliation, Calibration Alert and CIP Compliance Log - all approved into MVP in v2.2/v3.0 - move to Out of Scope (§22), not Post-MVP: not currently planned, revisit only if requested. §17's MVP table, §22, §23 and §24 are amended below, not deleted, so the v2.2/v3.0 reasoning stays traceable.

Changelog (v5.0): Rate Chart Reference upgraded from a read-only display to a live Rate and Amount CALCULATION per collection, ported from an existing reference implementation (legacy Android app: MilkCollectionFragment.getAmount(), RateFormulaFragment, branch offline-db) - see new §25 for the full formula spec. Scope boundary drawn precisely: computing Rate/Amount for a single collection, from already-configured rate settings, is now in MVP (§17); authoring/managing per-farmer rate charts and farmer/pourer SETTLEMENT (ledger, advances, incentives, periodic payment cycles) remain out of scope, unchanged from §20/§24 - see the amendment notes at both.

Changelog (v6.0): Documents a product decision found already implemented in code (this pass verified it against source, not assumed): the automatic ACCEPTED/HOLD/REJECTED suggestion computed by quality-rule validation is advisory only. The system still computes it, but the Result actually persisted on the transaction is the operator's own explicit decision - a human must always press the decision, the system never auto-applies its own suggestion. See the amendment notes under §10 and §18. This is a real change from this BRD's earlier framing (§1, §9, §12, §19's workflow diagrams showed "Validate Quality" flowing directly into "ACCEPT/HOLD/REJECT" without specifying who decides) - the diagrams' shape is still accurate, only the mechanics needed spelling out.

# Process Overview

The chilling centre process as scoped for the Windows application and cloud API MVP (v4.0): milk arrival through reception, quality decision, local save, and holding at the centre, with a read-only rate-chart reference and a notification hook attached to the steps they serve, syncing to a central cloud system for RBAC, audit and reporting. See §17 for the full MVP scope and §22 for what is explicitly out of scope.

# 1. Purpose

A native Windows application installed at each chilling centre will directly communicate with the weighing machine and milk analyser, maintain operational data locally, and synchronize transactions to the central cloud when connectivity is available.

Milk Arrival

↓

Source / Vehicle Selection

↓

Start Reception

↓

Read Weighing Machine

↓

Read Milk Analyser

↓

Combine Readings

↓

Validate Quality

↓

ACCEPT / HOLD / REJECT

↓

Save Locally

↓

Sync to Cloud

# 2. Architecture

CLOUD • API • PostgreSQL • RBAC • Reports •

↓  HTTPS / API

WINDOWS APPLICATION WPF UI • Reception Engine • Device Manager • Scale Adapter • Analyser Adapter • SQLite • Sync Engine • Audit Logger

↙                                      ↘

RS232                                      RS232

Weighing Machine                         Milk Analyser

# 3. Technology Stack

| Component | Choice |
| --- | --- |
| Desktop | C# / .NET 8+ / WPF |
| UI Pattern | MVVM |
| Local Database | SQLite |
| Persistence | Entity Framework Core or equivalent |
| Device Interface | RS232 / Serial |
| Cloud | HTTPS REST API |
| Cloud Database | PostgreSQL |

# 4. Application Modules

Login

Dashboard

Milk Reception

Sources

Vehicles

Transactions

Device Configuration

Device Testing

Synchronization Status

Local Database

Audit Logs

Application Settings

# 5. Device Configuration

## 5.1 Weighing Machine

| Setting | Example |
| --- | --- |
| Connection | RS232 |
| COM Port | COM4 |
| Baud Rate | 2400 |
| Data Bits | 8 |
| Stop Bits | 1 |
| Parity | None |
| Flow Control | None |
| Read Timeout | Configurable |
| Parser | Device-specific |

The currently verified scale configuration is 2400 baud. Baud rate and all serial parameters remain configurable.

## 5.2 Milk Analyser

| Setting | Value |
| --- | --- |
| Connection | RS232 |
| COM Port | Configurable |
| Baud Rate | Configurable |
| Data Bits | Configurable |
| Stop Bits | Configurable |
| Parity | Configurable |
| Flow Control | Configurable |
| Timeout | Configurable |
| Parser | Device-specific |

# 6. Device Test

Test Connection

↓

Open COM Port

↓

Receive Data

↓

Parse

↓

Display Connection + Reading Result

The UI shall distinguish port unavailable, disconnected device, connected/no data, invalid data, and successfully parsed data.

# 7. Device Parser Architecture

Raw RS232 Data

↓

Device-specific Parser / Adapter

↓

Normalized Reading

↓

Reception Workflow

Manufacturer-specific protocols remain isolated from the core reception workflow.

# 8. Critical Device Integration Rule

No device protocol shall be invented. Where documentation is unavailable, the protocol may be established from controlled raw captures from the physical device and must be documented and tested.

# 9. Milk Reception

Select Source

↓

Select Vehicle

↓

Start Reception

↓

READ DEVICES

↓

Review Scale + Quality Readings

↓

Validate

↓

Save Transaction

READ DEVICES Scale → Weight Analyser → FAT / SNF / CLR / Temperature

# 10. Transaction & Quality

Transactions shall contain a unique ID/number, date/time, source, vehicle, operator, quantity, quality parameters, device sources and acceptance status.

| Parameter | Minimum | Maximum |
| --- | --- | --- |
| FAT | 3.0 | 6.0 |
| SNF | 8.0 | 10.0 |
| Temperature | 0 °C | 10 °C |

Result: ACCEPTED / HOLD / REJECTED. Reasons for HOLD/REJECT shall be stored.

Amended (v6.0): quality-rule validation computes a suggested Result, but it is advisory only. The Result actually persisted is the operator's own explicit decision, made after reviewing the readings and the suggestion - the system never auto-applies its own suggestion. This applies to every "Validate"/"Validate Quality" step referenced elsewhere in this document (§1, §9, §12, §19).

Note (v2.1): Acidity, Clot-on-Boiling (COB) and antibiotic/adulteration testing are deferred, not in scope for now — no verified measurement instrument or protocol is currently available for these parameters. FAT/SNF/CLR/Temperature remain the quality gate. Revisit once a device/method is identified; see §22.

# 11. Local-First Database

Users / Roles / Permissions

Sources / Vehicles

Devices / Configurations

Milk Reception / Quality / Device Readings

Quality Rules

Audit Logs

Synchronization Records

Device Reading

↓

Validation

↓

SQLite Local Save

↓

Sync Queue

↓

HTTPS Cloud Synchronization

Internet connectivity must not be required to complete a physical milk reception.

# 12. Synchronization

Every local transaction shall have a unique idempotency key. Cloud retries must not create duplicates.

ONLINE • Pending Sync: 0 • Last Sync: 18:32:15

OFFLINE • Pending Sync: 12 • Last Successful Sync: 17:51:04

# 13. Windows Behaviour

Start with Windows where configured

Run without developer tooling

Recover from failures

Reconnect devices

Maintain local database

Operate offline

Synchronize automatically

Show clear device and cloud status

# 14. Security & Audit

HTTPS / TLS

Secure token handling

Secure local storage

RBAC

Least privilege

Audit logging

Centre-level authorization

No public exposure of physical devices

Audit records shall cover login/logout, transaction creation/modification/cancellation, device configuration, quality rules, manual entry, overrides, source/vehicle changes and synchronization.

# 15. Manual Fallback

Authorized users may enter readings manually when a device is unavailable. Such readings shall be explicitly marked Reading Source = MANUAL.

# 16. Reports

Daily reception

Source-wise

Vehicle-wise

Quality

Device status / communication

Exports: Excel, CSV and PDF, subject to permissions.

Note (v2.2): Dispatch and reconciliation reports are now part of the MVP. Chilling-tank, tank-stock and equipment-utilisation reports remain out of scope (§22).

# 17. MVP

| Area | Scope |
| --- | --- |
| Devices | Scale RS232; analyser RS232; COM/baud/serial configuration; testing; status |
| Reception | Source; vehicle; read devices; review; validate; accept/hold/reject; save |
| Local | SQLite; offline operation; history; audit; sync queue |
| Cloud | Authentication; transaction sync; idempotency; sync status |
| Source Hierarchy | Optional parent BMC/route code on Source - blank for a private CC, populated for cooperative-tier deployments |
| Rate Calculation | Live Rate & Amount calculation per collection from FAT/SNF/Weight against the configured Rate Formula (Fat-vs-SNF or TS-based) - see §25. Not farmer/pourer settlement (ledger, advances, payment cycles), which remains out of scope (§20). |
| Notifications | Outbound-notification hook/abstraction only; SMS gateway wiring deferred (§24) |

Note (v2.2): Outbound tanker dispatch and reconciliation/closing-stock reporting are now part of the MVP (moved from Post-MVP — see §23 and the v2.2 changelog above). Chilling-tank tracking, bulk storage tank telemetry, CIP and plant/equipment monitoring remain out of scope entirely (§22).

# 18. Acceptance Criteria

Physical scale connects using configured serial settings and produces the actual weight.

Milk analyser connects and provides supported quality readings.

One action reads configured devices and associates readings with one transaction.

Transactions are saved locally before synchronization.

Offline transactions are preserved and later synchronized without duplication.

Quality validation and ACCEPTED / HOLD / REJECTED decisions work.

Amended (v6.0): read as "quality validation computes a suggestion, and the operator's own explicit Accept/Hold/Reject decision is what gets persisted" - see §10.

RBAC, audit and privileged operations are enforced.

Device failures do not crash the application.

# 19. Final Solution Vision

LOGIN

↓

SELECT SOURCE

↓

SELECT VEHICLE

↓

START RECEPTION

↓

READ DEVICES

↓

VALIDATE QUALITY

↓

ACCEPT / HOLD / REJECT

↓

SAVE LOCALLY

↓

SYNC TO CLOUD

↓

RECEIPT / REPORT

Core principle: the Windows application owns local operation — device communication, parsing, reception, SQLite persistence and synchronization — while the cloud provides centralized storage, RBAC,and reporting.

# 20. Chilling Centre Scope Clarification

A Bulk Milk Cooler (BMC) is a village-level collection-and-cooling point — a weighing scale, an analyser, a 500–5,000 L tank — feeding a handful of routes. A Milk Chilling Centre (CC/MCC) sits a level above it: a regional facility that receives from several BMCs/VLCs, tests and reconciles what arrives, and dispatches onward by tanker to a processing plant. This application — Chilling Centre Milk Collection (CCMC) — is named for, and scoped to, the Chilling Centre tier.

Individual farmer-level pouring, per-farmer rate charts and farmer settlement belong to the upstream BMC/VLC tier and are explicitly out of scope for this application.

Amended (v5.0): “per-farmer rate charts” above means authoring/managing a farmer-specific pricing table, and “farmer settlement” means the ledger/advances/payment-cycle machinery - both remain out of scope. Computing a Rate and Amount for a single collection, against an already-configured Rate Formula, is a different and narrower thing, now in MVP - see §25.

## 20.1 BMC vs. Chilling Centre

| Aspect | BMC | Chilling Centre |
| --- | --- | --- |
| Primary purpose | Collect and rapidly cool milk | Receive, chill, store, test, consolidate and dispatch milk |
| Typical scale | Village / local collection point | Regional facility |
| Milk source | Farmers / VCCs | Multiple VCCs / BMCs |
| Storage | 500–5,000 L tank | Several thousand to tens of thousands of litres |
| Tanker dispatch | Usually none | Yes — to the processing plant |
| This application (CCMC) | Out of scope | In scope |

# 21. Comparable Existing Software

Before further build investment, six vendors selling software at the Milk Chilling Centre tier in India were reviewed directly against their own public product pages, not summarized secondhand. This is not greenfield ground — comparable software already exists and is in production use.

| Vendor / Product | Confidence | Coverage Summary |
| --- | --- | --- |
| Everest Instruments — Milk Reception Automation / RMRD | Verified | Explicitly targets Chilling Centres and BMCs: reception, rate charts, route/vehicle timing, sampling, testing, tanker dispatch, physical stock. |
| KVR Technologies — Dairy ERP | Verified | Named Chilling Centre module: pooled intake, weighment, quality check, milk stock, stock transfer to plant. |
| TecXpert — Dairy ERP | Verified | VLC → MCC → EWS/analyser testing → dispatch challan → plant receipt, with reconciliation against the challan. |
| Sort String — ProcuPort | Verified (cross-checked) | Cloud platform, Farmer → VLC → Chilling Centre → Head Office; names plant-intake reconciliation and live inventory. |
| Stellapps — SmartCC | Verified | Cloud analytics layer for chilling centres: quality/quantity mismatch detection, tanker prioritisation by milk age, remote rate-chart control. |
| NanoDairy | Verified | Chilling-centre listing exists but thin on public detail; core product is village/BMC-level. |

None of the six vendors' public materials claim chilling-tank/batch tracking, bulk storage tank telemetry, CIP, or plant/equipment monitoring — see §22 for the resulting scope decision.

# 22. Out of Scope (By Decision)

The following are explicitly not being built, by decision, not oversight:

Chilling-tank / batch tracking (tank assignment, inlet/outlet temperature, chilling start/end time, lot tracking)

Bulk storage tank telemetry (level, temperature, milk age, utilisation)

CIP telemetry — chemical usage, temperature/time curves, equipment cleaning history (unchanged, still out of scope). Note (v3.0): a separate, lightweight CIP compliance LOG (date/operator/result, no telemetry) is in MVP - see §17, §24. This is not a reversal of the telemetry decision, which stands.

Note (v4.0): the CIP compliance LOG referenced above has itself been moved to Out of Scope by business decision - see the four new bullets below and the v4.0 changelog. The telemetry exclusion in this bullet was never affected either way.

Outbound tanker dispatch to the processing plant (v4.0 - approved into MVP in v2.2, now moved back out by business decision; the underlying capability is unchanged from how §23/§24 described it, just not being built for now).

Reconciliation / closing-stock reporting (v4.0 - same status as dispatch, which it depends on; moved out together).

Device calibration tracking and due-date alerts (v4.0 - approved into MVP in v3.0, now moved back out by business decision).

CIP compliance log - record-keeping only, date/operator/result (v4.0 - approved into MVP in v3.0, now moved back out by business decision; distinct from CIP telemetry above, which was never in scope).

Plant/equipment monitoring (chiller, compressor, pumps, tank sensors, power/failure events)

Rationale: checked against all six vendors in §21 — including Stellapps SmartCC, a cloud layer purpose-built to monitor chilling centres — none publicly claim any of the above. Six independent vendors agreeing on the same boundary indicates a market pattern, not a gap specific to this BRD: tank/CIP/equipment telemetry is treated as a separate plant-automation/SCADA problem (sold by instrumentation/engineering firms alongside the physical reception dock), not part of the procurement/reception software layer this application belongs to.

Also deferred — not descoped, revisit once a measurement method exists, no fixed date: laboratory tests beyond FAT/SNF/CLR (acidity, Clot-on-Boiling, antibiotic/adulteration screening) and a lab workflow distinct from reception. No verified instrument or protocol for these parameters is available today; see the note under §10. Priority raised (v3.0): both the TUMUL RFP and Aavin's live field deployment confirm this is already-expected, not speculative — treat as the top priority the moment device/instrument work resumes.

# 23. Post-MVP Roadmap

Superseded again (v4.0): dispatch and reconciliation, described below as moved into MVP under v2.2, have since been moved to Out of Scope (§22) by business decision - not Post-MVP, not currently planned. This section is kept as-is beneath for traceability of the v2.2 reasoning; it no longer reflects the current MVP.

Superseded (v2.2): both capabilities below moved into MVP (§17) — see the v2.2 changelog. Kept here for the original in-scope-vs-descoped reasoning and the vendor evidence behind it, not as a current Post-MVP list.

Two capabilities are confirmed as in-scope for a Chilling Centre — and are proven, vendor-built features (§21) — but are sequenced after the MVP in §17, not included in it.

| Capability | Why In Scope | Why Post-MVP |
| --- | --- | --- |
| Outbound tanker dispatch to the plant | Every workflow diagram in this document (§1, §9, §12, §19) ends at “sync to cloud” with no outbound leg; multiple vendors (Everest, TecXpert) build this. | Depends on reception and offline-first sync being stable first. No new device/hardware required — a workflow and data addition only. |
| Reconciliation & closing stock | The day-to-day control a chilling-centre operator actually runs: received vs. dispatched, closing stock, variance. Built by TecXpert and ProcuPort. | Depends on dispatch data (above) existing first. Pure arithmetic over reception and dispatch quantities — no tank telemetry required. |

Sequencing (v2.2): within the MVP itself, build dispatch before reconciliation, since reconciliation depends on dispatch data existing. Both depend on reception and offline-first sync being stable first, which is already the case.

# 24. Final Scope Validation — Karnataka & Tamil Nadu (v3.0)

Validated against two lenses: a private, independently-owned chilling centre (this BRD's original target), and cooperative-federation-scale operations - Karnataka's KMF and Tamil Nadu's Aavin district unions. Evidence base: a full 171-page ERP RFP from TUMUL (Tumkur District Co-operative Milk Producers Societies' Union, KMF-affiliated, Karnataka), BAMUL's real operating scale (Bengaluru, Karnataka - 7 chilling centres, 14.0 LLPD, 246 bulk milk coolers), and Aavin's live-deployed stack (Tamil Nadu - cloud-connected milk analysers tendered at 1,397 locations, 45 BMCs already live in Madurai district, quality-based pricing already operating in the field). No public RFP exists for a private operator, structurally - private-lens validation rests on the six-vendor landscape (§21) instead.

Amended (v4.0): of the five additions this section approved into MVP, two - device calibration tracking and the CIP compliance log - have since been moved to Out of Scope (§22) by business decision, alongside dispatch and reconciliation. The remaining three (Source Hierarchy field, Rate Chart Reference, Notification Hook) stay in MVP. The validation and evidence below are kept as the historical record of that decision, not edited to match the current, narrower MVP.

Approved and added to MVP: CIP compliance logging (record only, not telemetry - §22 unchanged), device calibration tracking with due-date alerts, an optional parent-BMC/route field on Source (blank for a private CC), a read-only rate-chart reference at reception (no farmer-payment computation - confirmed this does not cross that boundary for how the product will be positioned), and an outbound-notification hook/abstraction (interface only, no SMS gateway wired in this pass). Dispatch and Reconciliation's MVP scope refined with tare/gross tanker weight and FAT/SNF variance tracking respectively.

Superseded in part (v5.0): the rate-chart reference described here as read-only now performs live Rate/Amount calculation - see §17, §20's amendment, and §25. This is still distinct from farmer/pourer settlement, which is unaffected and stays out of scope.

Explicitly considered and excluded from MVP: building a real SMS send-path (gateway account/integration) - deferred until a gateway is chosen, only the hook ships now. Offline-media (USB/pen-drive) sync fallback for zero-connectivity sites - low priority for the private-CC target, revisit only if a specific pilot site has no connectivity option at all. Chilling-tank/bulk-storage/plant-equipment telemetry, farmer/pourer payment computation, and full rate-chart authorship all remain out of scope, confirmed correct by both lenses (§22 unchanged).

Amended (v5.0): “farmer/pourer payment computation” here refers to farmer-level settlement (ledger, advances, incentives), which is still out of scope. Per-collection Rate/Amount calculation is a narrower, separate capability now in MVP - see §25.

This scope was reviewed and signed off; the BRD is considered finalized at v3.0 for the Windows application and cloud API. Hardware/device protocol work (Videocon scale, milk analyser) remains separately tracked and out of scope for this revision, per instruction.

# 25. Milk Rate Calculation — Business Logic

Source: MilkCollectionFragment.getAmount(), RateFormulaFragment (legacy Android reference implementation, branch offline-db). Ported here as the authoritative business logic for the Windows application's Rate Calculation module (§17).

Rate is recalculated live as soon as FAT, SNF, and Weight are entered on the milk collection screen. The formula used depends on the Rate Type configured on the Rate Formula screen, stored as PREFS_RATE_TYPE.

## 25.1 Inputs

| Field | Source | Description |
| --- | --- | --- |
| FAT | Entered per collection | Fat % of the milk sample |
| SNF | Entered per collection | Solids-Not-Fat % of the milk sample |
| Weight | Entered per collection | Quantity of milk (litres/kg) |
| Value1, Value2 | Rate Formula setting (PREFS_RATE_VALUE1, PREFS_RATE_VALUE2) | Two configured base-rate components, used only in Fat-vs-SNF mode |
| TS Rate | Rate Formula setting (PREFS_RATE_TS) | Flat rate per Total Solids point, used only in TS mode |

If FAT, SNF, or Weight is blank, Rate and Amount are both set to 0 and no calculation runs.

## 25.2 Mode 1: Fat-vs-SNF (PREFS_RATE_TYPE = “Fat_vs_SNF”)

Milk value is split into a fat-based component and an SNF-based component, each weighted against a combined base rate (Value1 + Value2), plus a fixed adjustment constant.

Rate = (Value1 + Value2) × 0.22 × (FAT / 100)

+ (Value1 + Value2) × 0.36 × (SNF / 100)

+ 0.32

Amount = Rate × Weight

0.22 — fat weighting factor applied to the combined base rate

0.36 — SNF weighting factor applied to the combined base rate

0.32 — fixed per-litre adjustment added to every calculation

Requires both Value1 and Value2 to be configured; otherwise Rate/Amount = 0

## 25.3 Mode 2: TS-based (any other PREFS_RATE_TYPE)

Total Solids (TS) is treated as the sum of FAT and SNF, and priced at a single flat rate per TS point.

TS = FAT + SNF

Rate = (TS × TS_Rate) / 100

Amount = Rate × Weight

Requires TS_Rate to be configured; otherwise Rate/Amount = 0

## 25.4 Output Formatting

Both Rate and Amount are rounded/formatted to 2 decimal places (##.##) before being displayed and used for printing/SMS receipts.

Scope note: this section defines only the per-collection Rate/Amount calculation. It does not include farmer/pourer ledgers, advances, incentive schemes, or periodic settlement cycles, all of which remain out of scope per §20.

Technical Design Document

Chilling Centre Milk Collection — Native Windows Application

Version 1.0  •  September 2026

# 1. Technical Objective

The Windows application is the local-first operational system. It communicates with devices, parses readings, runs reception, persists data locally and synchronizes with the cloud.

# 2. Technical Architecture

CLOUD + API + PostgreSQL + RBAC

↓ HTTPS

WINDOWS APPLICATION WPF → Application → Domain → Infrastructure SQLite • Serial • Device Adapters • HTTP • Sync

↙                                      ↘

RS232                                      RS232

Scale                                      Analyser

# 3. Solution Structure

CCMC.sln src/ ├── CCMC.Desktop/ WPF / ├── CCMC.Application/ Workflows / Services ├── CCMC.Domain/ Entities / Rules / Interfaces ├── CCMC.Infrastructure/ SQLite / Serial / Devices / HTTP / Logging └── CCMC.Contracts/ DTOs / API Contracts

# 4. Layer Responsibilities

| Layer | Responsibility |
| --- | --- |
| Presentation | WPF screens and operator interaction |
| Application | Reception, device orchestration, validation, synchronization |
| Domain | Transactions, readings, devices, rules |
| Infrastructure | SQLite, serial I/O, parsers, HTTP, logging, Windows integration |
| Contracts | Shared DTOs and API contracts |

# 5. Serial Pipeline

Serial Configuration

↓

Serial Connection

↓

Raw Bytes

↓

Frame Detection

↓

Device Parser

↓

Normalized Reading

Serial I/O must not block the WPF UI thread.

# 6. Serial Configuration Model

SerialConfiguration - ComPort - BaudRate - DataBits - StopBits - Parity - FlowControl - ReadTimeoutMs

Current verified scale: COM4 / 2400 / 8 data bits / 1 stop bit / no parity / no flow control.

# 7. Device Interfaces

IDevice ConnectAsync() DisconnectAsync() IsConnected IWeighingScale : IDevice ReadWeightAsync() IMilkAnalyser : IDevice ReadQualityAsync()

# 8. Adapter Architecture

IWeighingScale ↓ VideoconScaleAdapter ↓ WeightReading

IMilkAnalyser ↓ AnalyserAdapter ↓ MilkQualityReading

The reception workflow consumes normalized readings and contains no manufacturer-specific serial logic.

# 9. Videocon Scale Parser

Raw RS232 Bytes ↓ Frame Detection ↓ Videocon Protocol Parser ↓ Validation ↓ WeightReading

The actual protocol must not be guessed. It shall be derived from device documentation or controlled captures from the physical machine. Parser regression tests shall use captured real-device frames.

# 10. Reading Models

| Reading | Minimum Fields |
| --- | --- |
| WeightReading | Value, Unit, Stable, Timestamp, DeviceId, RawData |
| MilkQualityReading | FAT, SNF, CLR, Temperature, optional parameters, Timestamp, DeviceId, RawData |

# 11. Device Manager

Load configuration

Open/close COM ports

Detect failures

Reconnect

Expose device health

Route raw data to the correct parser

Each physical device must have one clear connection owner. No two application components may race to open the same COM port.

# 12. Reception Workflow

Select Source

↓

Select Vehicle

↓

Read Scale

↓

Read Analyser

↓

Normalize Readings

↓

Display / Review

↓

Validate Quality

↓

Persist Local Transaction

↓

Queue Sync

# 13. Local Persistence

Validated Reception

↓

SQLite Transaction

↓

Commit

↓

Create Sync Record

Business transaction and synchronization metadata should be persisted atomically wherever practical.

# 14. SQLite Schema

Users Roles Permissions UserRoles Sources Vehicles Devices DeviceConfigurations MilkReceptions MilkQuality DeviceReadings QualityRules AuditLogs SyncQueue SyncAttempts

# 15. Synchronization State Machine

PENDING → PROCESSING → SYNCED

Failure: PROCESSING → RETRY_WAIT → PROCESSING Permanent failure: PROCESSING → FAILED

# 16. Idempotency

Local Transaction + Unique Idempotency Key ↓ HTTPS Cloud Request ↓ Success OR Safe Duplicate Response

A lost response followed by a retry must not create a duplicate cloud transaction.

# 17. Cloud API Boundary

POST /api/v1/auth/login POST /api/v1/sync/receptions GET /api/v1/sources GET /api/v1/vehicles GET /api/v1/configuration

These are architectural examples only; the implementation must use the actual agreed cloud API contract.

# 18. Authentication & Authorization

Cloud Authentication

↓

Access Token

↓

Local Session

↓

RBAC-controlled Operations

Offline authentication requires an explicit security design. Credentials must not be stored insecurely just to enable offline operation.

# 19. Logging

| Category | Examples |
| --- | --- |
| Application | Startup, shutdown, exceptions |
| Device | Connect/disconnect/status |
| Serial | Configuration and receive errors |
| Parser | Raw message where appropriate, parse failures |
| Reception | Capture/save events |
| Synchronization | Attempts, success, retry, failure |
| Security | Authentication / authorization events |

# 20. Error Handling

COM port unavailable

Device disconnected

Malformed serial data

Invalid parser output

Cloud unavailable

Database transient failure

Synchronization failure

None of these conditions should crash the application.

# 21. Windows Deployment

Windows installer

No developer tools required

Required .NET runtime packaged or installed as prerequisite

Persistent application data and logs

Windows startup support

# 22. Background Operation

Windows ├── CCMC Background Service / Worker │ ├── Device monitoring │ └── Cloud synchronization │ └── CCMC Desktop └── WPF operator interface

If service + desktop are split, exactly one component must own each physical COM port.

# 23. Security

HTTPS/TLS

Secure token handling

Secure local storage

RBAC

Least privilege

Audit logging

No public device endpoints

Restricted device configuration

# 24. Data Integrity

Physical Reading

↓

Raw Reading

↓

Parsed Reading

↓

Reception Transaction

↓

Local SQLite

↓

Cloud Synchronization

The system should preserve enough provenance to determine device, timestamp, raw data where appropriate, parsed value, transaction and synchronization status.

# 25. Testing Strategy

| Level | Scope |
| --- | --- |
| Unit | Parsers, validation, state transitions, idempotency, configuration |
| Integration | Serial adapter → parser → reading → reception → SQLite |
| Hardware | Actual scale/analyser, stable readings, disconnect/reconnect |
| Offline | Internet loss, restart, power loss, reconnect, sync |
| UI | Reception, configuration, device test, error states |

# 26. Parser Development Workflow

Identify Device

↓

Establish Serial Settings

↓

Capture Raw Data

↓

Identify Framing

↓

Identify Fields

↓

Implement Parser

↓

Captured-Frame Tests

↓

Physical Device Test

↓

Document Protocol

↓

Register Adapter

# 27. Performance

Normal device capture should complete within a few seconds after valid readings are received. UI responsiveness must be preserved during serial I/O, reconnection, database operations and cloud synchronization.

# 28. Maintainability

Adding a new device model should normally require a new adapter/parser implementing the existing device interface. Reception, persistence, validation and synchronization should remain unchanged.

# 29. Engineering Rules

No fake device data in production reception.

No guessed device protocol.

Local save before cloud synchronization.

Cloud failure must not lose transactions.

Retries must be idempotent.

UI must not contain business logic.

Parser must not contain business logic.

One component owns a COM port.

Device configuration must not be hard-coded.

Every physical-device parser must have captured-frame regression tests.

# 30. Recommended MVP Build Order

Foundation (.NET/WPF/MVVM/SQLite/Logging)

↓

Serial Framework

↓

Actual Videocon Scale Adapter

↓

Milk Analyser Adapter

↓

Reception Workflow

↓

Offline Queue + Sync

↓

Cloud Integration

↓

Installer + Windows Hardening + Hardware Tests

# 31. Final Technical Architecture

CLOUD Web + API + PostgreSQL + RBAC + Reports

↓ HTTPS

WINDOWS APPLICATION WPF/MVVM → Application → Domain → Infrastructure SQLite • Serial • Device Adapters • Sync

↙                         ↘

Scale RS232              Analyser RS232

The Windows application owns local operation; the cloud provides centralized storage, security, dashboards and reporting.